| Issue | Tilt Complaint |
| --- | --- |
| Camera | NDE-xxxx-AL |
| Description | Rework Instructions |

Report

# Disassemble IR ASSY (IR LED + IR Holder): 3x M2x6 torx screw.

# Loosen the 2x side screws holding the lens: 2x M2 hex screw.

# Pull slightly the lens (do not pull it out completely) and push it back.

# Tighten the 2x side screws holding the lens: 2x M2 hex screw, 0,025Nm torque

# Re-assemble the IR ASSY (IR les + IR Holder): 3x M2x6 torx screw, 0,1Nm torque

BT-VS/PMO1